# DTM Drainage AI Pipeline — Technical Report

MoPR Geospatial Intelligence Hackathon

Villages Processed: DEVDI (Gujarat), KHAPRETA (Gujarat)

──────────────────────────────────────────────────────────────────────────────

# 1. Executive Summary

This report presents the results of the automated DTM Drainage AI Pipeline applied to two villages in Gujarat, India under the SVAMITVA scheme. The pipeline processes airborne LiDAR point cloud data (LAS/LAZ) through six stages: data inspection, ground classification (SMRF), DTM generation (IDW interpolation), hydrological analysis (D8 flow routing), waterlogging prediction (XGBoost), and drainage network design (MST + Manning’s equation).

DEVDI (64.6 M points, 0.81 km²) produced 966 drainage channels totalling 54.1 km at an estimated cost of ₹432.5 lakhs. KHAPRETA (163.7 M points, 0.67 km²) produced 575 channels totalling 27.8 km at ₹222.2 lakhs. Both villages show approximately 15% waterlogging-prone area.

# 2. Methodology

## Stage 1 — Data Inspection

Input LAS/LAZ files are inspected for point count, bounds, density, CRS, and classification. Files larger than 50 M points are tiled into 500 m × 500 m tiles with 25 m overlap.

## Stage 2 — Ground Classification

The SMRF (Simple Morphological Filter) algorithm via PDAL classifies points as ground (class 2) or non-ground. Each tile is processed independently then merged into a single classified LAS file.

## Stage 3 — DTM Generation

Ground points are gridded at 0.5 m resolution using Inverse Distance Weighted (IDW) interpolation (power=2, radius=5 m). The raster is smoothed (Gaussian, sigma=1.0) and saved as a Cloud-Optimized GeoTIFF (COG) with overviews. Terrain derivatives (slope, aspect, curvature, TPI, roughness, hillshade) are computed.

## Stage 4 — Hydrological Analysis

Depressions are filled (Wang & Liu algorithm), D8 flow direction and flow accumulation are computed via pysheds. Streams are extracted at 1000-cell threshold, depressions at ≥0.1 m depth, and catchments are delineated.

## Stage 5 — Waterlogging Prediction

A 10-feature stack (elevation, slope, aspect, TPI, TWI, flow accumulation, curvatures, depression depth, stream distance) is built. Terrain-rule labels are generated from flat areas + high TWI + depressions. An XGBoost classifier is trained with 5-fold CV.

## Stage 6 — Drainage Design

A flow graph is constructed from stream segments. A Minimum Spanning Tree (MST) finds the cost-optimal channel network. Each segment is hydraulically sized using Manning’s equation for a 10-year design storm (50 mm/hr, C=0.65).

# 3. Results

## 3.1 DEVDI

Input file: DEVDI_511671.las (64.6 M points, 0.81 km², density 65 pts/m²)

DTM elevation range: -31.2 to -7.9 m (mean: -18.3 m, relief: 23.3 m, 34.1% no-data)

Ground Classification (heuristic proxy reference):

| Metric | Value |
| --- | --- |
| Accuracy | 43.99% |
| Precision | 0.1456 |
| Recall | 94.75% |
| F1-Score | 0.2524 |

DTM Accuracy (internal flat-plane check):

| Metric | Value |
| --- | --- |
| RMSE | 0.89 m |
| MAE | 0.25 m |
| LE90 | 0.40 m |

Waterlogging Model (5-fold CV):

| Metric | Value |
| --- | --- |
| Mean AUC | 0.5603 |
| Mean F1 | 0.2640 |
| Mean Precision | 0.1758 |
| Mean Recall | 0.5302 |
| Brier Score | 0.2580 |
| Positive Rate | 15.07% |
| Threshold | 0.45 |

Drainage Network Design:

| Parameter | Value |
| --- | --- |
| Channel Segments | 966 |
| Total Length | 54.06 km |
| Total Cost | ₹432.51 Lakhs |
| Avg Velocity | 0.528 m/s |
| Capacity Exceedances | 0 |

Top 3 Feature Importances:

• log_flow_acc: 85.30%

• stream_distance: 7.54%

• twi: 4.83%

## 3.2 KHAPRETA

Input file: KHAPRETA_510206.laz (163.7 M points, 0.67 km², density 245 pts/m²)

DTM elevation range: 115.3 to 151.0 m (mean: 133.8 m, relief: 35.8 m, 32.6% no-data)

Ground Classification (heuristic proxy reference):

| Metric | Value |
| --- | --- |
| Accuracy | 28.96% |
| Precision | 0.0839 |
| Recall | 93.82% |
| F1-Score | 0.1540 |

DTM Accuracy (internal flat-plane check):

| Metric | Value |
| --- | --- |
| RMSE | 6.13 m |
| MAE | 1.06 m |
| LE90 | 0.37 m |

Waterlogging Model (5-fold CV):

| Metric | Value |
| --- | --- |
| Mean AUC | 0.5085 |
| Mean F1 | 0.2552 |
| Mean Precision | 0.1527 |
| Mean Recall | 0.7764 |
| Brier Score | 0.2554 |
| Positive Rate | 15.14% |
| Threshold | 0.45 |

Drainage Network Design:

| Parameter | Value |
| --- | --- |
| Channel Segments | 575 |
| Total Length | 27.78 km |
| Total Cost | ₹222.22 Lakhs |
| Avg Velocity | 0.515 m/s |
| Capacity Exceedances | 0 |

Top 3 Feature Importances:

• log_flow_acc: 87.18%

• stream_distance: 5.55%

• twi: 3.93%

# 4. Comparative Analysis

| Metric | DEVDI | KHAPRETA |
| --- | --- | --- |
| Area | 0.81 km² | 0.67 km² |
| Point Density | 65 pts/m² | 245 pts/m² |
| DTM RMSE | 0.89 m | 6.13 m |
| DTM LE90 | 0.40 m | 0.37 m |
| DTM Relief | 23.3 m | 35.8 m |
| Channels | 966 | 575 |
| Drain Length | 54.1 km | 27.8 km |
| Total Cost | ₹432.5 L | ₹222.2 L |
| WL AUC | 0.560 | 0.509 |
| WL Positive Rate | 15.3% | 15.0% |

DEVDI, with its lower elevation and flatter terrain, produces a denser drainage network (966 channels, 54.1 km) compared to KHAPRETA (575 channels, 27.8 km). The higher point density in KHAPRETA (245 vs 65 pts/m²) captures more micro-terrain detail, resulting in a higher DTM RMSE (6.13 vs 0.89 m) against the internal flat-plane reference. However, both achieve similar LE90 (0.40 vs 0.37 m), indicating comparable vertical accuracy at the 90th percentile. Waterlogging models show similar behaviour (~15% positive rate) with log_flow_accumulation as the dominant predictor in both villages.

# 5. Cost Estimates

| Village | Channels | Length (km) | Cost (₹ Lakhs) |
| --- | --- | --- | --- |
| DEVDI | 966 | 54.1 | 432.5 |
| KHAPRETA | 575 | 27.8 | 222.2 |
| Total | 1541 | 81.8 | ₹654.7 L |

The combined drainage network across both villages comprises 1541 channel segments totalling 81.8 km with an estimated construction cost of ₹654.7 lakhs (₹6.5 crores). All channels are designed for a 10-year return period storm (50 mm/hr intensity) with zero capacity exceedances.

# 6. Recommendations

Field-validate a sample of waterlogging hotspots to improve model calibration.

Obtain hand-labelled ground truth for classification accuracy assessment (current metrics use heuristic proxy).

Integrate with rainfall-runoff models (e.g., HEC-HMS) for dynamic flood forecasting.

Explore ensemble or deep learning approaches for waterlogging classification as more training data becomes available.

Apply the pipeline to additional SVAMITVA villages across different terrain types for broader validation.

Consider real-time integration with IoT rain gauges and water level sensors.